Mopar small block - 408ci - all new parts except for seasoned 360 block. Professionally built and tuned on Superflow 902 dynamometer. This pump gas street motor is designed for the long cruise or a trip to the track. Makes 517HP & 504TQ. These are not magazine HP numbers, these are real.  9.94:1 compression ratio

Engine includes complete documentation:

- 50+ pictures taken during build – thumb drive included

- All part numbers of every part in the engine & cost in Excel spreadsheet

- All build specifications – flow numbers /cam specs /bob weight card/compression calculations.

- Torque specs and clearances, camshaft degree specs, etc.

- Video of engine on dyno (on thumb drive)

Description:

Block
seasoned 360 block bored to 4.030 (.030 over) with Scat 4.00 stroke crank. main studs w/windage tray & custom baffle, KB forged pistons, Mahle rings, brass freeze plugs, new balancer, Viton rear seal, new distributor gear bushing, new cam bearings, Scat I beam rods with ARP bolts,  new HV oil pump, new pick-up, new oil pan. Brass ball valves. new oil slinger. Head studs, true roller chain, MP timing chain tensioner. Honed with torque plate. 

Heads
CNC Ported Aluminum heads (w/flow sheet) flow CFM 282 in./ 214 Ex., 2.02 and 1.60 valves. Everything new, set up for the custom Hughes flat tappet camshaft, chrome moly retainers & 10 degree locks.  Autolite plugs, Felpro 1008 head gaskets. Sheetmetal valve covers – polished. valve cover studs.  2 breathers.  Head studs.

Valvetrain
Special lifters ($200+) w/ .016 hole in bottom to pressure oil cam lobes, Stainless steel 1.6 roller rockers, shafts have studs-12pt nuts, custom pushrods, Hughes custom Flat tappet camshaft,  Duration @.050 250/254 w/ .624/.640 gross lift.  Billet oil fill in valve cover.  Billet 9 keyway true roller timing chain & MP chain tensioner. Hardened distributor gear.

Induction
Edelbrock Victor 340 intake manifold.  SRE 750 DP carb with 4-corner idle, adjustable air bleeds, clear sight bowls, billet metering blocks and billet base plate, 1" carb spacer, carb studs, 

Electrical & other
MSD billet distributor, MSD wires, wire looms, Autolite race plugs, complete engine documentation. Joe Gibbs break-in oil used. This motor prefers a 1-3/4" tube quality header. 

We have other complete new MOPAR motors also, big and small block, all HP levels. We only use quality parts and all engines are dynoed and tuned. No leaks. We do ship engines in the USA.