MOPAR “Torque Ziller” pump gas RB big block - 452ci – If you desire a street friendly pump gas motor with unprecedented torque at a reasonable price. read on.

All new parts except for seasoned 440 block.  Professionally built and tuned on Superflow 902 dynamometer.  9.41:1 compression.  This 440 has a very flat high torque curve that averages 500 ft pounds from 3000 – 5000rpm.  It peaks at 516 at 4200 yet has 490 at 3000 and 475 @ 5000.  This motor will pull, pull and keep pulling.  Nothing higher than a 3.23 rear gear is recommended.

Description:
Block
seasoned 440 block (71) hot tanked, bored to 4.380, brass deep freeze plugs, brass ball valves,  std./std. forged crankshaft 3.74 stroke, ARP rod bolts, forged pistons 4.380, assembly balanced, new oil pan 8QT, HV oil pump, new pick-up, windage tray, 3 bolt Torrington bearing true roller billet timing chain, chrome dip stick, file fit rings, MP front & rear seals.

Heads
440 Source aluminum heads ported by Wright Racing in Georgia. (w/flow sheet) 2.14 and 1.88 valves. ,sheetmetal & chromed valve covers with breathers.  Springs set up for custom Hughes camshaft.  ARP head bolts are used.

Valvetrain
440 Source 1.5 roller rockers – billet hold downs, Custom Smith Brothers  pushrods, Comp Cams flat tappet lifters, hardened distributor gear, new bronze bushing, custom distributor hold down, billet electronic distributor, Hughes Flat tappet camshaft  238/242 @ .050  .558/.564 lift.  9 keyway billet Torrington bearing timing chain. 

Induction
Edelbrock Torker intake manifold – ported matched.  No carb included at this price. Dyno carb was used, will buy new in box Proform 850 DP carb with 4-corner idle, adjustable air bleeds, billet base plate. (add $600)  Includes 1/2" carb spacer, carb studs.  We can add any manufactures EFI system.   

Other
All engine hardware is nice.  Aluminum water pump housing.  Electric pump in pictures is for the dyno, motor comes with aluminum belt driven water pump.  8.8mm wires, wire looms, Nice chrome breathers, complete engine documentation. Every part number from all parts used included, cylinder head flow numbers, cam card, build sheet,and dyno sheets. Joe Gibbs break-in oil used.

We have other complete new MOPAR motors also, big and small block, all HP levels. We only use quality parts and all engines are dynoed and tuned. No leaks. We do ship engines in the USA.   We also have race motors in stock ready, just call.